MEDIA SANITIZATION SOP

Doc ID: TAG-OT-SOP-03

Reference: IACS UR E27 §4.5 (Supply Chain Security)

Applicability: All Third-Party External Media (USB, SSD, Laptops)

1. Purpose

To establish a mandatory procedure for the verification and sanitization of any external digital media before it is connected to a vessel's System Under Consideration (SuC). This prevents the "cross-contamination" of malware from shore-side or other vessel environments.

2. Scope of Equipment

This procedure applies to:

- OEM Service Laptops: Even if used only for diagnostics.

- Removable Media: USB flash drives, SD cards, and external hard drives containing firmware or software updates.

- Diagnostic Hardware: Proprietary communication interfaces and VCIs.

3. Pre-Boarding Verification (Step 1)

- OEM Certificate: Review the "Cleanliness Certificate" or scan log requested in the Pre-Arrival Letter (TAG-OT-VND-02).

- Scan Recency: Verify that the antivirus scan date is within 24 hours of the technician's arrival.

4. Physical Boarding & Inspection (Step 2)

- Inventory: The ETO must document the serial number of every device the technician brings across the gangway in the Sanitization Checklist (TAG-OT-VND-03).

- Physical Integrity: Inspect devices for unauthorized hardware (e.g., "Rubber Ducky" USB injectors or hidden wireless transceivers).

5. Onboard Scanning Procedure (Step 3)

If the vessel possesses a Cyber-Security Kiosk (Pillar C):

- Insert the vendor's media into the isolated kiosk.

- Perform a full heuristic scan using at least two different AV engines.

- Once cleared, the kiosk will generate a "Clean" report to be filed with the Service Entry Permit.

If no hardware kiosk is available (Manual Method):

- Connect the media to a Stand-Alone (Air-Gapped) laptop dedicated for scanning.

- Manually trigger a deep scan of the drive letter.

- DO NOT open files directly from the drive during the scan.

6. Connection & Authorization (Step 4)

- VLAN Assignment: Once sanitized, connect the device ONLY to the Service VLAN.

- Witness Rule: The ETO should witness the technician's first connection to ensure no unauthorized data transfer or "any-to-any" firewall rules are requested.

7. Post-Service De-registration (Step 5)

- Clean Exit: Verify that no temporary files or "shadow software" (e.g., remote access tools) remain on the ship's systems.

- Permit Closure: Finalize the Service Entry Permit (TAG-OT-SEP-01) by recording the disconnect time and signing off on the audit.